# HuaweiSA QODN/Vumatel FTTH Auto Splicing Plan

Dear Huawei FTTH Planner,

This plugin will save you long hours or even days of manual Excel work. Instead of building splicing diagrams by hand, this plugin auto-generates the full Excel splicing plan with all DC segments, splitter legs, pole numbers, and house connections.

## What It Does

Creates a professional Excel splicing diagram showing:

- DC cable segments for each DJ (DCxx.1, DCxx.2, DCxx.3, DCxx.4)

- Splitter legs (1-8) for each connected house

- Pole numbers where each DJ is installed

- House names with coordinates (LAT/LONG)

- Block and AG grouping

## Simple Inputs (What You Need)

Before running, confirm these layers exist and are correctly labeled:

- ✅ Be sure you confirm the labeling is correct by the Labeling plugin first

- ✅ DJ Polygons — one polygon per DJ (with Block field)

- ✅ DJ Points — DJ locations (with name, position, splitter, dc_name fields)

- ✅ House Points — all premises (with name field)

- ✅ Pole Points — all poles (with name field)

- Drop Lines (optional) — fallback for houses outside DJ polygons

- DC Lines (optional) — for FJ tracing in FEEDER column

- FJ Points (optional) — for FJ name lookup

## ⚠️ CRITICAL — Use GeoPackage Format

Use .gpkg layer/project format to avoid the issue of float numbers from .shp files.

| Format | Position Field | Result |
| --- | --- | --- |
| GeoPackage (.gpkg) ✅ | Integer (1, 2, 3, 4) | 4 DC columns ✓ |
| Shapefile (.shp) ❌ | Real (1.0, 2.0, 3.0, 4.0) | Only 1 DC column ✗ |

If you only have Shapefile, either: - Convert to GeoPackage: Right-click layer → Export → Save Features As → GeoPackage - Or use plugin v8.6.22+ which auto-converts Real → Integer

## How to Run

- Open QGIS → Processing Toolbox → FTTH Tools → FTTH Auto Splicing Plan

- Select each layer from the dropdown

- Set Snap Tolerance (default 5m is usually fine)

- Choose Output folder for the Excel file

- Click Run

- Open the generated Excel file

## ⚠️ Check the Output

- Each DJ should have its own DC segment column (DCxx.1, DCxx.2, etc.)

- Each house should show splitter leg number (1-8)

- If you see only 1 DC column: check your DJ Points position field is 1,2,3,4 (not all 1)

By Mustafa M M Ellaham